# Dynamic Positioning

# FAILURE MODE & EFFECTS ANALYSIS
GC80 Gyro
With: K-Pos DP21 – HiPAP - cJoy

This document is available under the Creative Commons Zero v1.0 license, as you downloaded and opened this document, it is acceptance of the license terms and conditions.

The main thrust of this license is to protect ALUCIANT in terms of warranty and liability, we do not warrant the templates nor accept liability for any of the templates and their use.

More details can be found on the ALUCIANT website at www.aluciant.com/cc0license/

Gyro Compasses

Description, Location and Redundancy Concept

The vessel is fitted with three Simrad GC80 gyro compasses, all of which can be selected into DP. The DPO can enable all three into DP at any time. The DP system will then monitor and compare the selected gyros and alarm for heading difference and power loss.

The gyro compasses are probably the most critical of the vessel reference sensors, failure of the vessel heading reference can cause the vessel to oscillate or lose the heading altogether with serious consequences if the vessel is near a platform.

All three gyro units feed to the DPC RSER modules using an NMEA 0183 sentence. Heading inputs are also required for the HIPAP systems. Gyro 3 heading data is fed to HiPAP 1, also providing survey feed and the gyro repeaters on the bridge wing. Gyro 1 supplies the independent joystick system with a heading reference.

The GC80 unit should withstand short power failures without excessive deviation, although in practice this may not be the case. A two-degree deviation could be expected after a three-minute failure. Once power is re-established the compass should quickly find the correct heading, although monitoring of the system is recommended prior to being selected as heading solution. Latitude error is virtually eliminated due to the liquid damping system used in the gimbals array.

There is a facility for inputting the vessel’s latitude and speed manually to each individual gyro. There is also the option of updating the position automatically from the DGNSS (RS422, GGA or GLL NMEA 0183 telegram). [TBC]

The gyro compasses are located as follows:

- No. 1 Gyro – [LOCATION]

- No. 2 Gyro – [LOCATION]

- No. 3 Gyro – [LOCATION]

Power supplies for the gyro compasses are as follows: [TBC]

- No. 1 Gyro – UPS 1 – XX & 24Vdc from XX

- No. 2 Gyro – UPS 2 – XX & 24Vdc from XX

- No. 3 Gyro – UPS 2 – XX

Configuration for DP

All three Gyros should be verified as being closely aligned and selected into the DP system. It is the operator’s decision which should be used as the preference.

Failure Modes & Effects of the Gyro Compasses

Significant failure modes of the gyro compasses are as follows:

- Gyro Power Supply Failure:
This would cause alarms to sound on the DP OS. The faulty gyro would be automatically deselected, and the two remaining gyros would be used. The DP system would continue functioning with just two gyros; however, no voting would take place.

- Gyro giving an Incorrect Reading:
During DP operations all three gyros should be in use. Should one of the three gyro readings deviate beyond a pre-defined limit from the others (usually around 2°) then the [DP system] will reject the faulty gyro as a solution after three erroneous readings are received. The DP will then continue to use the remaining two healthy gyros as reference. Should a further gyro deviate from the heading then a “difference alarm” will be issued alerting the DPO to the problem. In this instance it would be the DPOs responsibility to select the healthy gyro. If one of the two gyros signal become ‘frozen’, the faulty gyro is not rejected, but an alarm is activated, and the signal is given zero weighting automatically by the DP control system.

- Loss of DGNSS Signal to Gyro:
This will not result in the immediate rejection of a gyro from the DP control system. However, there may be a heading difference introduced from the actual heading due to the lack of position data available to the gyro. As each gyro is supplied with a DGNSS signal from a different DGNSS, this will only affect one gyro. Using the manual input values for latitude and speed as opposed to the DGPS input to a gyro would prevent this failure effect occurring. [TBC]

Hidden Failures of the Gyro Compasses

There are no apparent hidden failures in the gyros.

However, it should be noted that on any failure of a gyro the DP system will not automatically re-select the gyro as a heading solution if the unit becomes available. The DPO must be aware of the selection of healthy gyro signals to the DP.

Common Mode Failures of the Gyro Compasses

There are no apparent common mode failures for the gyro compasses.

System Configuration that could Defeat the Redundancy of the Gyro Compasses

Setting the manual input value for vessels’ latitude incorrectly into a gyro compass can cause a discrepancy between the readings on that gyro and one with the correct latitude value input. This can cause unexpected deselection of a gyro by the [DP SYSTEM] discrepancy function (with 3 or more gyros online) or ‘Gyro difference’ alarms if two gyros are online. With a single gyro online, that being the incorrectly configured one, there is the potential for the vessel heading to be incorrect. Watch checklists and standard operating procedures should enable the DPO to identify this error. This error will not exceed the WCFDI.

Setting the manual input value for vessels speed incorrectly into a gyro compass can cause a discrepancy between the readings on that gyro and one with the correct speed value input. This can cause unexpected deselection of a gyro by the [DP SYSTEM] discrepancy function (with 3 or more gyros online) or ‘Gyro difference’ alarms if two gyros are online. With a single gyro online, that being the incorrectly configured one, there is the potential for the vessel heading to be incorrect. Watch checklists and standard operating procedures should enable the DPO to identify this error. This error will not exceed the WCFDI.

Maloperation of the Gyro Compasses

Maloperation of the gyro compasses could be failure to select all three gyros into DP. The most serious act of maloperation would be to select only one gyro leaving the vessel with no redundancy. Should the selected gyro fail then the DPO would have to promptly select a healthy gyro. With experienced and competent operators onboard, this is considered unlikely. On DP, as long as there is one gyro input, there will be no alarms to state that there is only one Gyro selected into the system.

Worst Case Failure of the Gyro Compasses

Worst case failure of the gyro system would be if only one gyro was selected into the DP system and was supplying incorrect data to the DPC-2 and HiPAP system. If this was the case, then the DPO could select one of the other gyros and DP operations could continue.

Transient effect on position: This failure may cause some position excursion.

Detection: Audible and visual alarms at DP operator stations.