# Dynamic Positioning

# FAILURE MODE & EFFECTS ANALYSIS	
Inertial Labs - MRUs

This document is available under the Creative Commons Zero v1.0 license, as you downloaded and opened this document, it is acceptance of the license terms and conditions.

The main thrust of this license is to protect ALUCIANT in terms of warranty and liability, we do not warrant the templates nor accept liability for any of the templates and their use.

More details can be found on the ALUCIANT website at www.aluciant.com/cc0license/

Motion Reference Units

Description, Location and Redundancy Concept

There are three MRUs fitted onboard. Two are Inertial Labs MRU-B1.1 2-axis units which measure pitch and roll, with the third an Inertial Labs MRU-B2 3-axis unit which measures heave, pitch, and roll of the vessel. Magnetic fields pick up the movement of the vessel, voltages are induced and amplified to give output signals proportional to the pitch, roll and heave.

Various reference systems such as the DGNSS and HiPAP use accurately measured angles relative to the hull for their inputs to DP. The heave measurements are particularly required for the HiPAP systems. The MRU input to DP is used to correct the DGNSS signals taking into consideration the angle of the hull, fore and aft, and port and starboard, relative to the horizontal. The transponder positions detected by the HiPAP systems are corrected for vessel pitch, roll and heave by the APC 12 computer before the corrected position is transmitted to the DP. The importance of the MRU data increases as the weather worsens and the ship movements become more severe.

Each MRU is alarmed at the DP Console for pitch/roll difference and loss of power.

Failure of one MRU will cause the units rejection by the DP system; with measurement input to the systems being maintained using the healthy units.

MRUs 1 and 2 are powered via the [SOURCE]. MRU 3 is powered by the [SOURCE]. The MRUs require 24VDC.

MRU 3 interfaces to HiPAP OS via a serial line Splitter which is powered from [SOURCE].

All three MRU’s are installed inside a protective box mounted on the centerline of the winch room on the aft bulkhead

Power supplies for the MRUs (via the DPCs) are as follows:

- No. 1 MRU - DPC, A-side of cabinet (UPS 1)

- No. 2 MRU - DPC, B-side of cabinet (UPS 2)

- No. 3 MRU – [SOURCE]

Configuration for DP

All three MRU’s should be selected into the DP system. It is the operator’s decision which should be used as the preference.

Failure Modes & Effects of the Motion Reference Units

Significant failure modes of the MRU are considered to be:

- Power Supply Failure:
For one MRU this would cause alarms to sound on the DP OSs. The faulty MRU would be automatically deselected, and the remaining MRU’s would be used.

- MRU giving Incorrect Reading:
If the system is only using two MRU’s then there will be no voting system in place but if there is an error, there will be a difference alarm on the DP OS. Correct selection of MRU’s by the DPO’s would mitigate this.

- Failure of the Signal Input to the DP SYSTEM:
From the MRU to DP would lead to the faulty MRU being deselected. This would have the same effect as loss of power where the [DP SYSTEM] will continue to function using the healthy MRU.

- Failure of the Serial Line Splitter:
It will lead to loss of the interface to related HiPAP. Failure of the MRU signal to HiPAP will result in HiPAP being rejected from the DP desk. It will not affect DP operation as DP continues to use rest of reference systems.

Hidden Failures of the Motion Reference Units

Hidden failures of the MRUs concern incorrect data being passed to the [DP SYSTEM], this would only occur if only one or two MRUs were being used as there needs to be three units online to allow automatic rejection by the DP control system.

Additionally, if the mounting angle in the MRU software has not been correctly input, or if all the MRUs on board were given initially wrong roll and pitch angles during the commissioning stage then there could be adverse implications in the way the controllers try to compensate for external forces in determining the ships position based on the reference systems (GPS) available.

The above two hidden failures are particularly important on vessels where the DGNSS antenna is mounted on a high reference point (DGNSS 1 and DGNSS 2 on the main mast). Take for instance the example when two of the three MRUs have been correctly set up with the right mounting angle offsets and the third MRU were not. Typically, the MRU that is not set up correctly will be rejected from the DP system due to the voting in the DP software. However, should one of the two MRUs that have been correctly set up fail and the DP system believes that the incorrectly set up MRU is to be “in use” then, there is a possibility of the DP system creating a new set point a few meters away and the vessel reacting accordingly. There is also the possibility of all three MRUs set up with incorrect roll and pitch relative to the vessels actual roll and pitch. During periods of calm weather, the effects of the error may not be obvious. However, should there be rough weather whilst the vessel is on DP, the vessel may find difficulty in maintaining position. To mitigate this, operators must include monitoring of MRU data as a routine operation. Occasional referencing with other available sensors should also be part of the DPO’s checklist. The MRUs should be verified during the Customer Acceptance Trials (CAT), preferably whilst alongside and calm weather.

Common Mode Failures of the Motion Reference Units

Once installed and commissioned correctly there are no apparent common mode failures of the Motion Reference Units.

System Configuration that could Defeat the redundancy of the Motion Reference Units

With only one MRU selected into DP, a loss of that MRU will potentially result in the loss of input signals to other PMS (e.g., HIPAP), resulting in a degradation of their accuracy and performance, with the potential for rejection from the DP control system. Normal procedure during DP operations will be to have all three MRUs selected into DP with the outputs split between the PMS such that a loss of one MRU will not result in the loss of all PMS. The use of DP check lists should mitigate any configuration errors.

Maloperation of the Motion Reference Units

Once installed and commissioned correctly there are no maloperation errors identifiable.

Worst Case Failure of the Motion Reference Units

Worst case failure of the MRU system would be loss of power to an MRU. This would result in the loss of that MRU. The DP control system would utilize the remaining MRUs to maintain position of the vessel. This failure could also cause a loss of signal to the acoustic system (dependent on the configuration selected). This does not exceed the WCFDI.

Transient effect on position: No effect on position.

Detection: Audible and visual alarms at DP operator stations.